# upfbench — Runbook

Setup, dependencies, and how to run the test suites

coRAN Labs

June 2026

Table of Contents

A step-by-step, dependency-complete guide to running upfbench against an SD-Core BESS-UPF. Everything here was validated end-to-end on this VM (Ubuntu 22.04, single virtio NIC, SD-Core BESS-UPF in af_packet mode).

TL;DR: deploy the UPF → install host deps → build pfcpsim → (optional) install LaTeX → edit configs/sdcore-bess.yaml → python3 -m upfbench.cli run --config ... --suite all.

## 0. What the framework does

upfbench points at a 5G UPF, runs a test suite, and produces a standards-aligned report. Three suites:

| Suite | What it measures | Needs |
| --- | --- | --- |
| performance | single-tunnel throughput (NDR/PDR), latency/jitter, burst, multi-flow | pybess (rules) + tcpreplay (GTP-U) |
| load | many UEs: session capacity, per-UE throughput, latency-under-load | pfcpsim (N sessions) + tcpreplay (per-TEID GTP-U) |
| pfcp | N4/PFCP conformance per 3GPP TS 29.244 (pass/fail matrix) | pfcpsim only (no traffic) |

It connects to an already-running UPF (it does not deploy the UPF).

## 1. Prerequisites (in order)

### 1.1 — The UPF must be deployed and healthy

Deploy SD-Core + BESS-UPF in af_packet mode via Aether OnRamp (see ~/SDCORE_AETHER_ONRAMP_DEPLOY.md). Confirm it’s up:

export KUBECONFIG=$HOME/.kube/config
export PATH=$PATH:/var/lib/rancher/rke2/bin
kubectl get pod upf-0 -n aether-5gc          # expect: upf-0  5/5  Running
kubectl exec -n aether-5gc upf-0 -c bessd -- bessctl show port | grep -E "accessFast|coreFast"

The host must reach the pod’s access interface (the af_packet N3 ingress):

ping -c2 -I access 192.168.252.3             # pod access IP; expect 0% loss

### 1.2 — kubectl access

The adapter and pfcpsim control shell into the pod via kubectl. Make sure: - kubectl is on PATH (RKE2 puts it in /var/lib/rancher/rke2/bin), - KUBECONFIG points at a config that can reach the cluster (~/.kube/config).

### 1.3 — Passwordless sudo (for the traffic suites)

The traffic generator runs sudo tcpreplay and sudo taskset. The user running upfbench needs passwordless sudo (the pfcp suite alone does NOT need sudo).

### 1.4 — Host packages

sudo apt-get update
sudo apt-get install -y python3 python3-pip tcpreplay

### 1.5 — Python dependencies

pip3 install --user "PyYAML>=6.0" "Jinja2>=3.1" scapy

- PyYAML, Jinja2 — config + report rendering (declared in pyproject.toml).

- scapy — the generator crafts GTP-U packets into a pcap (Suites 1 & 2).

### 1.6 — Go + build pfcpsim (Suites 2 & 3 only)

pfcpsim is vendored as Go source in third_party/pfcpsim/; build the two binaries. It needs Go ≥ 1.25 (Ubuntu’s apt Go is too old — install from the tarball):

# install a current Go
cd /tmp && curl -sLO https://go.dev/dl/go1.26.4.linux-amd64.tar.gz
sudo rm -rf /usr/local/go && sudo tar -C /usr/local -xzf go1.26.4.linux-amd64.tar.gz
export PATH=$PATH:/usr/local/go/bin

# build pfcpsim + pfcpctl
cd ~/Shiva/upf-benchmark-framework/third_party/pfcpsim
GOFLAGS=-mod=mod GOTOOLCHAIN=local go build -o pfcpsim ./cmd/pfcpsim
GOFLAGS=-mod=mod GOTOOLCHAIN=local go build -o pfcpctl ./cmd/pfcpctl
ls -l pfcpsim pfcpctl     # two binaries; gitignored (build locally per host)

### 1.7 — LaTeX (optional, for PDF reports)

Without it, reports are written as .tex (still complete); with it you get .pdf.

sudo apt-get install -y texlive-latex-base texlive-latex-recommended \
                        texlive-latex-extra texlive-fonts-recommended

## 2. Install the framework

Pure-Python, no build needed. Run it as a module (always works):

cd ~/Shiva/upf-benchmark-framework
export KUBECONFIG=$HOME/.kube/config
export PATH=$PATH:/var/lib/rancher/rke2/bin:$HOME/.local/bin:/usr/local/go/bin

python3 -m upfbench.cli list      # sanity check: lists suites + test cases

(Optional: pip install -e . gives an upfbench command, but needs setuptools ≥ 64.)

## 3. Configure the campaign

Edit configs/sdcore-bess.yaml. Key fields:

campaign: UPF-BM-2026-001          # output dir name under campaigns/
sut: { cpu: ..., nic: ..., kernel: ... }   # static facts; adapter overlays live image/mode/ports

upf:
  adapter: sdcore_bess
  n3_iface: access                 # BESS access port (N3 ingress)
  n6_iface: core                   # BESS core port (N6 egress)
  namespace: aether-5gc            # k8s location of the UPF pod
  pod: upf-0
  bessd_container: bessd
  gen_iface: access                # host interface the generator sends from
  n3_remote_ip: "192.168.252.3"    # UPF N3 IP (MAC auto-read from the pod)
  gen_cpu_affinity: auto           # pin generator OFF bessd's worker core (~3x effect)

suite: performance                 # default suite; override with --suite

performance:                       # Suite 1 knobs
  control: pybess
  generator: tcpreplay
  frame_sizes: [128, 512, 1024, 1518]
  trial_duration_s: 5              # raise (e.g. 30-60) for publication-grade stability
  max_rate_mpps: 0.12              # NDR/PDR search ceiling
  ...
load:                              # Suite 2 knobs
  control: pfcpsim
  generator: tcpreplay
  ue_counts: [10, 100, 1000, 5000] # LT-01 capacity ramp
  lt02_ue_count: 100               # LT-02 throughput UE count
  lt03_ue_counts: [1, 10, 100]     # LT-03 latency-vs-load points
  ...
pfcp:                              # Suite 3 (no extra knobs needed)
  procedures: [association, establish, modify, delete, error_handling]

# baseline: campaigns/UPF-BM-2026-000/results.json   # uncomment for a comparison section

## 4. Run it

cd ~/Shiva/upf-benchmark-framework
export KUBECONFIG=$HOME/.kube/config
export PATH=$PATH:/var/lib/rancher/rke2/bin:$HOME/.local/bin

# list suites + test cases
python3 -m upfbench.cli list

# run one suite
python3 -m upfbench.cli run --config configs/sdcore-bess.yaml --suite performance
python3 -m upfbench.cli run --config configs/sdcore-bess.yaml --suite load
python3 -m upfbench.cli run --config configs/sdcore-bess.yaml --suite pfcp

# run everything in one campaign
python3 -m upfbench.cli run --config configs/sdcore-bess.yaml --suite all

# interactive menu
python3 -m upfbench.cli

Approx. run times on this VM (default knobs): performance ~3–4 min, load ~2–3 min, pfcp ~30 s, all ~6–7 min.

## 5. What each suite does (and what it produces)

performance — installs a wildcard “forward-all” rule via pybess, then tcpreplay blasts GTP-U from the host into the UPF’s access socket: - TC-01 NDR/PDR per frame size (binary search), TC-03 in-pipeline latency, TC-04 burst (drops/drain), TC-08 multi-flow. TC-02 bidirectional is skipped (needs a downlink path — see §8).

load — pfcpsim installs N real per-UE PFCP sessions; tcpreplay drives GTP-U on the matching per-UE TEIDs: - LT-01 session capacity ramp, LT-02 aggregate + per-UE forwarding verification, LT-03 latency vs UE count.

pfcp — pfcpsim drives each N4 procedure and asserts the response: - CF-01 association, CF-02 establish, CF-03 modify, CF-04 delete, CF-05 unknown-SEID rejection. Output is a pass/fail matrix.

## 6. Outputs

campaigns/<campaign-id>/
  results.json            # machine-readable: KPIs, per-test tables, captured commands
  report-performance.pdf  # one report per suite that ran (.tex if no LaTeX)
  report-load.pdf
  report-pfcp.pdf
  raw/                    # pybess scripts, pfcpsim server log, etc.

## 7. Operational notes (read these)

- Start from a clean UPF for trustworthy numbers. Heavy session churn degrades the pfcp-agent↔bessd datapath over a long session. If results look wrong (e.g. 0 forwarded), reset the pod:

- kubectl delete pod upf-0 -n aether-5gc
kubectl wait --for=condition=ready pod/upf-0 -n aether-5gc --timeout=120s

- datapath down auto-recovery: the pfcpsim control detects this and restarts the pfcp-agent + re-associates automatically — no action needed mid-run.

- CPU affinity matters (~3×): gen_cpu_affinity: auto keeps the software generator off bessd’s worker core. Leave it on auto.

- af_packet is non-monotonic: the no-drop rate (TC-01) can exceed the heavy-overload rate (TC-04) because the kernel socket livelocks under saturation. Numbers are also noisy at these low rates — raise trial_duration_s for stable, publication-grade results.

- TRex vs tcpreplay: this VM has no DPDK NIC, so we use tcpreplay. On real DPDK hardware, set generator: trex (the plugin is wired) for higher rates + per-flow stats.

## 8. Troubleshooting

| Symptom | Cause / fix |
| --- | --- |
| kubectl exec ... command not found | KUBECONFIG/PATH not set (see §2 exports) |
| Generator sends but 0 forwarded | UPF datapath degraded — reset the pod (§7) |
| tcpreplay: command not found / permission | install tcpreplay; ensure passwordless sudo |
| pfcpsim could not connect / datapath down | auto-recovered; if persistent, reset the pod |
| pfcpsim build fails on Go version | install Go ≥ 1.25 from the tarball (§1.6) |
| Report stays .tex (no PDF) | LaTeX not installed (§1.7) — .tex is still complete |
| TC-01 numbers vary run-to-run | af_packet low-rate noise — raise trial_duration_s |

## 9. What’s left / roadmap

Working today (validated end-to-end): all three suites against SD-Core af_packet — performance (TC-01/03/04/08), load (LT-01/02/03 incl. per-UE forwarding verification), pfcp conformance (CF-01..05), plus per-suite PDF reports with baseline comparison.

Deferred / future: 1. TC-02 bidirectional — needs a real downlink GTP-U path (comes naturally with TRex, or a DL session + GTP-encap setup on this VM). Currently skipped. 2. True per-UE fairness (LT-02) — current per-UE is the fair-share average plus an individual forwarding check; true under-load per-UE share needs BESS FlowMeasure per-flow counters (blocked on a multi-worker “leader flip” restriction). 3. Worker auto-tune (TC-05) — auto-pick the best bessd worker count/core pinning; limited on this VM (chart caps at 2 workers, contended cores) — more valuable on dedicated isolated-core hardware. 4. More UPF adapters — OAI-UPF / Open5GS / free5GC / eUPF (drop a new adapters/<x>.py; the suites don’t change). 5. TRex generator — implement traffic/trex.py for high-rate + per-flow stats on DPDK hardware.

Validated: 2026-06-14, SD-Core BESS-UPF af_packet (upf-bess rel-2.4.3), single-NIC VM.